# 🏢 3-Tier Enterprise Network Lab

## 🎯 Objectives

- ✅ OSPF Area 0 between Core and Distribution layers

- ✅ Layer 3 routed EtherChannel (LACP) — Core ↔ Distribution

- ✅ Layer 2 EtherChannel + 802.1Q trunking — Distribution ↔ Access

- ✅ HSRP first-hop redundancy (DIST1=Active for VLANs 10/30, DIST2=Active for VLANs 20/40)

- ✅ Deterministic STP root placement per VLAN, aligned with the active HSRP gateway

- ✅ VTP version 2 for VLAN propagation

- ✅ VLAN segmentation: VLANs 10, 20, 30, 40

- ✅ L2 Security: BPDU Guard, port-security, DHCP snooping

- ✅ Named ACL restricting inter-VLAN management access

## 🗺️ Topology Overview

┌─────────────────────────────────────────────┐
│              CORE LAYER                     │
│   CORE1 ─────────────────────── CORE2       │
│   (1.1.1.1)   10.0.12.16/30   (2.2.2.2)    │
└──────┬──────────────────────────┬───────────┘
  Po10/Po11                    Po10/Po11
  (L3 EtherChannel)         (L3 EtherChannel)
┌──────┴──────────────────────────┴───────────┐
│           DISTRIBUTION LAYER                │
│   DIST1 ◄──── HSRP ────► DIST2              │
│ (Active: VLANs 10,30)  (Active: VLANs 20,40)│
└──────┬──────────────────────────┬───────────┘
     L2 EtherChannel trunks (LACP, dot1Q)
  ┌────┴────┬──────────┬──────────┴────┐
 [ACC1]  [ACC2]     [ACC3]       [ACC4]
VLAN10  VLAN20    VLAN30       VLAN40
  |       |          |             |
[PC1]   [PC2]      [PC3]        [PC4]

10.0.10.11  10.0.20.11  10.0.30.11  10.0.40.11

## 📦 IP Addressing Plan

### L3 EtherChannel Port-Channels (Routed)

| Link | Subnet | CORE IP | DIST IP |
| --- | --- | --- | --- |
| CORE1 ↔ DIST1 (Po10) | 10.0.12.0/30 | 10.0.12.1 | 10.0.12.2 |
| CORE1 ↔ DIST2 (Po11) | 10.0.12.4/30 | 10.0.12.5 | 10.0.12.6 |
| CORE2 ↔ DIST1 (Po10) | 10.0.12.8/30 | 10.0.12.9 | 10.0.12.10 |
| CORE2 ↔ DIST2 (Po11) | 10.0.12.12/30 | 10.0.12.13 | 10.0.12.14 |
| CORE1 ↔ CORE2 | 10.0.12.16/30 | 10.0.12.17 | 10.0.12.18 |

### Loopbacks (OSPF Router-IDs)

| Device | Loopback0 |
| --- | --- |
| CORE1 | 1.1.1.1/32 |
| CORE2 | 2.2.2.2/32 |
| DIST1 | 3.3.3.3/32 |
| DIST2 | 4.4.4.4/32 |

### VLAN SVIs & HSRP (on DIST1 and DIST2)

| VLAN | Name | Subnet | DIST1 SVI | DIST2 SVI | HSRP VIP | Active |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | DATA | 10.0.10.0/24 | 10.0.10.2/24 | 10.0.10.3/24 | 10.0.10.1 | DIST1 |
| 20 | GUSESTS | 10.0.20.0/24 | 10.0.20.2/24 | 10.0.20.3/24 | 10.0.20.1 | DIST2 |
| 30 | MGMT | 10.0.30.0/24 | 10.0.30.2/24 | 10.0.30.3/24 | 10.0.30.1 | DIST1 |
| 40 | SERVERS | 10.0.40.0/24 | 10.0.40.2/24 | 10.0.40.3/24 | 10.0.40.1 | DIST2 |

### Host Addresses

| Host | VLAN | IP | Gateway |
| --- | --- | --- | --- |
| PC1 | 10 | 10.0.10.11/24 | 10.0.10.1 (HSRP VIP) |
| PC2 | 20 | 10.0.20.11/24 | 10.0.20.1 (HSRP VIP) |
| PC3 | 30 | 10.0.30.11/24 | 10.0.30.1 (HSRP VIP) |
| PC4 | 40 | 10.0.40.11/24 | 10.0.40.1 (HSRP VIP) |

## DHCP Server (CORE1)

CORE1 acts as the centralized DHCP server for all four VLANs. DIST1 and DIST2 relay client broadcasts to it via ip helper-address (pointing at CORE1’s reachable address) configured on each VLAN SVI. The low end of each subnet is excluded to reserve room for the HSRP VIP and gateway addresses.

| Pool Name | VLAN | Subnet | Excluded Range | DNS Server | Domain |
| --- | --- | --- | --- | --- | --- |
| DATA | 10 | 10.0.10.0/24 | 10.0.10.1 - 10.0.10.10 | 10.0.40.12 | local.lab |
| GUESTS | 20 | 10.0.20.0/24 | 10.0.20.1 - 10.0.20.10 | 10.0.40.12 | local.lab |
| MGMT | 30 | 10.0.30.0/24 | 10.0.30.1 - 10.0.30.10 | 10.0.40.12 | local.lab |
| SERVERS | 40 | 10.0.40.0/24 | 10.0.40.1 - 10.0.40.10 | 10.0.40.12 | local.lab |

## 📋 Lab Tasks

### Phase 1 — Foundation

- Configure hostnames on all 8 network devices

- Set enable secret and console/VTY passwords

- Configure Loopback0 on CORE1 (1.1.1.1/32), CORE2 (2.2.2.2/32), DIST1 (3.3.3.3/32), DIST2 (4.4.4.4/32)

### Phase 2 — VTP & VLAN Propagation

- Configure VTP version 2, domain ENTERPRISE on all switches

- Set DIST1 as VTP Server; DIST2 and all Access switches as VTP Clients

- Create VLANs 10 (DATA), 20 (GUESTS), 30 (MGMT), 40 (SERVERS) on DIST1

- Verify propagation: show vlan brief on all access switches

### Phase 3 — STP Root Placement

- Configure DIST1 as primary root for VLANs 10 and 30 (matching its active HSRP role)

- Configure DIST2 as primary root for VLANs 20 and 40 (matching its active HSRP role)

- Enable PortFast and BPDU Guard on ACC1–ACC4 host-facing ports (Ethernet1/0)

- Verify: show spanning-tree vlan 10 — DIST1 must be root bridge for VLAN 10

### Phase 4 — L2 EtherChannel (Distribution ↔ Access)

- Configure LACP mode active port-channels on all Distribution–Access uplinks

- DIST1 side: Po1 per ACC switch using Et1/x and Et2/x interfaces per mapping table

- DIST2 side: Po2 per ACC switch using Et1/x and Et2/x interfaces per mapping table

- ACC side: Po1 toward DIST1 (Et0/0 + Et0/1), Po2 toward DIST2 (Et0/2 + Et0/3)

- Configure dot1Q trunking on all port-channels; allow only the VLAN for that access switch

- Verify: show etherchannel summary — Po1 and Po2 must show (P) bundled

### Phase 5 — L3 EtherChannel (Core ↔ Distribution)

- Configure all Core–Distribution member interfaces as routed (no switchport)

- Build LACP active port-channels: CORE1 Port-channel1↔DIST1, CORE1 Port-channel2↔DIST2, CORE2 Port-channel1↔DIST1, CORE2 Port-channel2↔DIST2 (CORE1 and CORE2 each number independently, starting from 1)

- Assign /30 IP addresses to the port-channel interfaces per the addressing plan

- Configure the direct routed link CORE1 Gi5 ↔ CORE2 Gi5 (10.0.12.16/30)

- Verify: show interfaces port-channel X — up/up; ping each /30 peer

### Phase 6 — OSPF Area 0

- Enable OSPF process 1 on CORE1, CORE2, DIST1, DIST2

- Advertise all routed interfaces (port-channels + direct link) and Loopback0

- Set passive-interface on all VLAN SVI interfaces at Distribution layer

### Phase 7 — HSRP

- On DIST1: configure HSRP groups 10 and 30 with priority 110 (Active); groups 20 and 40 stay at default priority (Standby)

- On DIST2: configure HSRP groups 20 and 40 with priority 110 (Active); groups 10 and 30 stay at default priority (Standby)

- VIPs: VLAN10=10.0.10.1, VLAN20=10.0.20.1, VLAN30=10.0.30.1, VLAN40=10.0.40.1

- Enable preempt on the active groups on each distribution switch

- Verify: show standby brief — DIST1 Active for VLANs 10/30, DIST2 Active for VLANs 20/40

### Phase 8 — L2 Security

- Enable DHCP snooping on ACC1–ACC4 for the appropriate VLANs

- Mark Distribution uplink port-channels as trusted; Ethernet1/0 (PC ports) as untrusted

- Configure port-security on Ethernet1/0 of all ACC switches: max 2 MACs, violation shutdown

- Verify: show ip dhcp snooping binding, show port-security interface Ethernet1/0

### Phase 9 — ACLs (Inter-VLAN Restriction)

- Create named ACL MGMT-RESTRICT on both DIST1 and DIST2

- Deny traffic sourced from 10.0.10.0/24 (VLAN 10) destined to 10.0.30.0/24 (VLAN 30 MGMT)

- Deny traffic sourced from 10.0.20.0/24 (VLAN 20) destined to 10.0.30.0/24 (VLAN 30 MGMT)

- Deny traffic sourced from 10.0.40.0/24 (VLAN 40) destined to 10.0.30.0/24 (VLAN 30 MGMT)

- Permit all other traffic

- Apply inbound on VLAN 10,VLAN 20,VLAN 40 SVI interfaces

- Verify: show ip access-lists MGMT-RESTRICT; test with ping from PC1/PC2 to PC3

### Phase 10 — End-to-End Validation

- ✅ PC1 ping PC2 (inter-VLAN — should succeed)

- ❌ PC1 ping PC3 (ACL block — should fail)

- ✅ PC1 ping PC4 (inter-VLAN — should succeed)

- Shut DIST1 → verify HSRP Active role for VLANs 10/30 moves to DIST2

- Remove one EtherChannel member link → verify traffic continues uninterrupted

- Confirm STP root bridge is unchanged after failures

## 📋 Per-Device Configuration Checklist

| Device | What to Configure |
| --- | --- |
| CORE1 | Hostname, Loopback0 (1.1.1.1/32), routed Port-channel1 (Gi1+Gi2) IP 10.0.12.1/30 toward DIST1, routed Port-channel2 (Gi3+Gi4) IP 10.0.12.5/30 toward DIST2, Gi5 IP 10.0.12.17/30 direct link to CORE2, OSPF process 1 area 0 on all (actual device uses GigabitEthernet interface naming, not Ethernet0/x). CORE1 also acts as the centralized DHCP server for all four VLANs — see DHCP Server section below. |
| CORE2 | Hostname, Loopback0 (2.2.2.2/32), routed Port-channel1 (Gi1+Gi2) IP 10.0.12.9/30 toward DIST1, routed Port-channel2 (Gi3+Gi4) IP 10.0.12.13/30 toward DIST2, Gi5 IP 10.0.12.18/30 direct link to CORE1, OSPF process 1 area 0 on all (CORE1 and CORE2 each independently use Port-channel1/2 — these are not globally unique numbers across devices) |
| DIST1 | Hostname, Loopback0 (3.3.3.3/32), VTP Server, VLANs 10/20/30/40, STP primary root for VLANs 10 & 30, routed Po10 (Et0/0+Et0/1) IP 10.0.12.2/30 toward CORE1, routed Po11 (Et0/2+Et0/3) IP 10.0.12.10/30 toward CORE2, L2 Po1 per ACC switch (trunks), SVIs for VLANs 10-40, HSRP active priority 110 on groups 10 & 30, OSPF area 0, ACL MGMT-RESTRICT inbound on VLAN 10,20,40 SVIs |
| DIST2 | Hostname, Loopback0 (4.4.4.4/32), VTP Client, STP primary root for VLANs 20 & 40, routed Po10 (Et0/0+Et0/1) IP 10.0.12.6/30 toward CORE1, routed Po11 (Et0/2+Et0/3) IP 10.0.12.14/30 toward CORE2, L2 Po2 per ACC switch (trunks), SVIs for VLANs 10-40, HSRP active priority 110 on groups 20 & 40, OSPF area 0, ACL MGMT-RESTRICT inbound on VLAN 10,20,40 SVIs |
| ACC1 | Hostname, VTP Client, LACP Po1 (Et0/0+Et0/1) trunk to DIST1, LACP Po2 (Et0/2+Et0/3) trunk to DIST2, allow VLAN 10 on trunks, Et1/0 = access VLAN 10 + PortFast + BPDU Guard, DHCP snooping (trust Po1+Po2), port-security on Et1/0 |
| ACC2 | Same as ACC1 but VLAN 20 |
| ACC3 | Same as ACC1 but VLAN 30 |
| ACC4 | Same as ACC1 but VLAN 40 |
| PC1–4 | Set IP per host addressing table; default gateway = HSRP VIP of respective VLAN |

## ✅ Success Criteria

- VLANs 10/20/30/40 visible on all switches via VTP (show vlan brief)

- DIST1 is STP root for VLANs 10 & 30; DIST2 is STP root for VLANs 20 & 40; BPDU Guard active on host ports

- All 8 port-channels show (P) bundled in show etherchannel summary

- OSPF adjacencies FULL on all Core–Distribution links

- HSRP: DIST1 Active (priority 110) for VLANs 10 & 30, DIST2 Active (priority 110) for VLANs 20 & 40

- DHCP snooping active; trusted ports correctly set

- Port-security active on all host-facing ports

- MGMT-RESTRICT ACL blocks VLAN 10,20,40 → VLAN 30

- PC1↔PC2 ping succeeds; PC1↔PC3 ping fails

- HSRP failover and EtherChannel link removal tests pass